VMware Cloud Foundation 9.1.1 · KB 441333

VCF Automation 正常移除

Step-by-Step 實作手冊

使用官方腳本 cleanup_component.py · 全程實測截圖與指令輸出

實測環境:home.lab 巢狀實驗室 · 2026-10-07

# 目錄

一、摘要

二、三條指令各碰到哪裡(流程圖)

三、環境與前提

四、Step 0 — 移除前狀態

五、Step 1 — 把腳本傳到 SDDC Manager

六、Step 2 — 確認 VSP 認證帳號

七、Step 3 — 列出要刪的元件 ID

八、Step 4 — ① 刪除 VCF Automation 元件

九、Step 5 — ② 刪除 Migration service engine 元件

十、Step 6 — 在 SDDC Manager 取得 root

十一、Step 7 — ③ 刪除 VCFA services runtime

十二、Step 8 — 移除後驗證

十三、注意事項與坑

附錄 A:可直接複製的完整指令

附錄 B:腳本參數一覽

# 一、摘要

KB 441333 提供官方腳本 cleanup_component.py,用來把 VCF Automation(VCFA)從 VCF fleet 中正常移除。本文在一套全新部署的 VCF 9.1.1(greenfield,含 VCFA)上完整執行一次,逐步記錄每條指令的輸出、UI 畫面與前後狀態。

| 步驟 | 指令 | 執行身分 | 實測耗時 | 結果 |
| --- | --- | --- | --- | --- |
| ① | delete vsp-component(VCF Automation) | vcf | 23 分 03 秒 | ✅ |
| ② | delete vsp-component(Migration service engine) | vcf | 12 分 02 秒 | ✅ |
| ③ | delete vsp-cluster(VCFA services runtime) | root | 3 分 10 秒 | ✅ |

結果:Fleet 元件由 10 個變 9 個,VCFA runtime VM(vcf-m03-auto-platform-lmh4w)已關機並刪除,SDDC Manager platform DB 的 vsp_cluster 紀錄已清除,VCFA console 連線逾時。Identity broker、VCF Operations、NSX、vCenter、SDDC Manager 完全不受影響。

## 三個最重要的注意事項

- VSP 認證帳號必須用 admin,不是 administrator@vsphere.local(後者回 HTTP 401 access_denied)。

- 是「三步」不是兩步:VCFA runtime 上還住著 Migration service engine(VCD_MIGRATOR),必須在刪 runtime 之前單獨刪掉。腳本不會幫你檢查。

- 刪 runtime(③)必須在 SDDC Manager 上以 root 執行;vcf 帳號不在 sudoers,su 又需要 TTY。

# 二、三條指令各碰到哪裡

圖 1　三條指令分別作用在 Fleet、管理 VSP、VCFA runtime、vCenter、SDDC Manager DB 的情形

# 三、環境與前提

| 角色 | FQDN | IP |
| --- | --- | --- |
| SDDC Manager(執行腳本的地方) | vcf-m03-sddcm01.home.lab | 10.0.1.65 |
| Fleet lifecycle | vcf-m03-fleet01.home.lab | 10.0.1.67 |
| VCF services runtime(管理 VSP) | vcf-m03-vsp01.home.lab | VIP .69,節點 .82/.83/.84 |
| VCFA services runtime(consumption VSP) | vcf-m03-auto-platform.home.lab | VIP .73,節點 .98 |
| VCF Automation console | vcf-m03-auto-vip.home.lab | 10.0.1.72 |
| vCenter | vcf-m03-vc01.home.lab | 10.0.1.60 |

## 前提

- VCF 9.1.1,VCFA 版本 9.1.1.0.25714559(≥ 9.1 才會真的刪除實體;< 9.1 只會清 Fleet 帳面)。

- 能以 vcf 帳號 SSH 進 SDDC Manager,並知道 root 密碼。

- 知道兩個 VSP 的 admin 密碼,以及 vCenter 的 administrator@vsphere.local 密碼。

- 本次 VCFA 尚未啟用 Supervisor(provider 首頁顯示 Pre-requisites for setup are missing)。

# 四、Step 0 — 移除前狀態

先把移除前的狀態完整記下來,事後才有東西對照。

圖 2　移除前:VCFA provider 可正常登入(admin / System Administrator)

圖 3　移除前:VCF Operations → Build → Lifecycle → Components,共 10 個元件,VCF Automation 9.1.1.0.25714559 在列

# 移除前狀態 — 2026-10-07 11:34:32

      
## vCenter(vcf-m03-vc01)上的 VM
  vcf-m03-auto-platform-lmh4w                      24 vCPU   96 GB  10.0.1.98    poweredOn
  vcf-m03-lic01                                     2 vCPU    4 GB  10.0.1.62    poweredOn
  vcf-m03-nsx01a                                    6 vCPU   24 GB  10.0.1.64    poweredOn
  vcf-m03-ops01                                     4 vCPU   16 GB  10.0.1.61    poweredOn
  vcf-m03-opsc01                                    4 vCPU   16 GB  10.0.1.66    poweredOn
  vcf-m03-sddcm01                                   4 vCPU   16 GB  10.0.1.65    poweredOn
  vcf-m03-vc01                                      4 vCPU   21 GB  10.0.1.60    poweredOn
  vcf-m03-vsp01-fz8hf                               4 vCPU   10 GB  10.0.1.82    poweredOn
  vcf-m03-vsp01-g7m86                              12 vCPU   24 GB  10.0.1.83    poweredOn
  vcf-m03-vsp01-kdwcz                              12 vCPU   24 GB  10.0.1.84    poweredOn
  vcf-services-runtime-template-9.1.1.0.25714471    6 vCPU   24 GB  -            poweredOff

      
## 端點
  https://vcf-m03-auto-vip.home.lab/automation/      200
  https://vcf-m03-auto-vip.home.lab/provider         301
  https://vcf-m03-auto-platform.home.lab/            404
  https://vcf-m03-vsp01.home.lab/                    404
  https://vcf-m03-fleet01.home.lab/                  404

      
## DNS
  vcf-m03-auto-vip           10.0.1.72
  vcf-m03-auto-platform      10.0.1.73
  vcf-m03-vsp01              10.0.1.69
  vcf-m03-fleet01            10.0.1.67

# 五、Step 1 — 把腳本傳到 SDDC Manager

腳本要在 SDDC Manager 上執行(③ 會直接操作 SDDC Manager 的本機資料庫)。傳完比對 sha256。

# Step 1 — 把 cleanup_component.py 傳到 SDDC Manager
# 本機 sha256:
253bf30d0b33e0c6640aa6cbdf96de41a4a6d75d0265db269b1f44099308ca4c */e/9.1/cleanup_component.py

      
$ pscp cleanup_component.py vcf@vcf-m03-sddcm01.home.lab:/home/vcf/
cleanup_component.py      | 4 kB |   4.0 kB/s | ETA: 00:00:09 |   9%cleanup_component.py      | 40 kB |  40.6 kB/s | ETA: 00:00:00 | 100%

      
$ ls -l /home/vcf/cleanup_component.py ; sha256sum /home/vcf/cleanup_component.py
-rw-r--r-- 1 vcf vcf 41591 Oct  7 03:33 /home/vcf/cleanup_component.py
253bf30d0b33e0c6640aa6cbdf96de41a4a6d75d0265db269b1f44099308ca4c  /home/vcf/cleanup_component.py

# 六、Step 2 — 確認 VSP 認證帳號

腳本對 VSP 的認證是 POST https://<vsp>/api/v1/identity/token(form-urlencoded,grant_type=password)。直接打一次,確認哪個帳號拿得到 token:

# Step 2 — 驗 VSP 認證帳號(從 SDDC Manager 打 /api/v1/identity/token)

      
vcf-m03-vsp01.home.lab           admin                          HTTP 200  ✅ 拿到 access_token
vcf-m03-vsp01.home.lab           administrator@vsphere.local    HTTP 401  ❌ {"error":"access_denied","error_description":"invalid credentials. incorrect credentials"}
vcf-m03-auto-platform.home.lab   admin                          HTTP 200  ✅ 拿到 access_token
vcf-m03-auto-platform.home.lab   administrator@vsphere.local    HTTP 401  ❌ {"error":"access_denied","error_description":"invalid credentials. incorrect credentials"}

只有 admin 拿得到 token;administrator@vsphere.local 兩個 VSP 都回 401 access_denied。之前曾因此誤判「官方腳本需要 VSP 認證、無法執行」,其實只是帳號用錯。

腳本會在任何刪除動作之前先驗證帳密(validate_vsp_credentials),帳號錯不會刪到一半。

# 七、Step 3 — 列出要刪的元件 ID

list 是唯讀操作,用 vcf 帳號即可。

$ python3 cleanup_component.py list vsp-component \
    -ff vcf-m03-fleet01.home.lab -vsrf vcf-m03-vsp01.home.lab -vsru admin -vsrp ********

      
Authenticating with VSP platform (vcf-m03-vsp01.home.lab)...
Authentication successful.
Fetching VSP components from Fleet...

      
Available VSP Components:
------------------------------------------------------------------------------------------------
ID | Type | Deployment | Version | FQDN | VCF Services Runtime FQDN
------------------------------------------------------------------------------------------------
12f80ada-c354-4b4d-9a19-6318ce58fe49 | VCF Automation | VSP | 9.1.1.0.25714559 | vcf-m03-auto-vip.home.lab | vcf-m03-auto-platform.home.lab
fe04c14c-4c68-4db4-b91b-05006adf5139 | Identity broker | VSP | 9.1.1.0.25679886 | vcf-m03-vidb.home.lab | vcf-m03-vsp01.home.lab
9c064f80-f259-4c50-98a8-97fbccf26cdc | VCD_MIGRATOR | VSP | 9.1.1.0.25714559 | N/A | vcf-m03-auto-platform.home.lab
------------------------------------------------------------------------------------------------

$ python3 cleanup_component.py list vsp-cluster \
    -ff vcf-m03-fleet01.home.lab -vsrf vcf-m03-vsp01.home.lab -vsru admin -vsrp ********

      
Authenticating with VSP platform (vcf-m03-vsp01.home.lab)...
Authentication successful.
Fetching VSP components from Fleet...

      
Available VSP Components:
------------------------------------------------------------------------------------------------
ID | Type | Deployment | Version | FQDN | VCF Services Runtime FQDN
------------------------------------------------------------------------------------------------
4fde9186-f3d4-48d6-ab64-a06664dad047 | VCF services runtime | VSP | 9.1.1.0.25714471 | vcf-m03-auto-platform.home.lab | vcf-m03-auto-platform.home.lab
------------------------------------------------------------------------------------------------

| ID | 類型 | 跑在哪個 runtime | 處理 |
| --- | --- | --- | --- |
| 12f80ada-…-6318ce58fe49 | VCF Automation | vcf-m03-auto-platform | ① 刪除 |
| 9c064f80-…-97fbccf26cdc | VCD_MIGRATOR(Migration service engine) | vcf-m03-auto-platform | ② 刪除 |
| 4fde9186-…-a06664dad047 | VCF services runtime(VCFA 用) | — | ③ 刪除 |
| fe04c14c-…-05006adf5139 | Identity broker | vcf-m03-vsp01(管理 VSP) | ⛔ 不能碰 |

VCD_MIGRATOR 也住在 VCFA runtime 上。它是 VCFA 部署時一起裝的 Migration service engine,必須在刪 runtime 之前另外刪掉。

# 八、Step 4 — ① 刪除 VCF Automation 元件

要同時給管理 VSP(-vsrf)與 VCFA runtime(-varf)的帳密,腳本才會連 VCFA runtime 上的元件一起處理。

# Step ① — 刪除 VCF Automation 元件  2026-10-07 11:37:29
$ python3 cleanup_component.py delete vsp-component \
    -c 12f80ada-c354-4b4d-9a19-6318ce58fe49 \
    -ff vcf-m03-fleet01.home.lab \
    -vsrf vcf-m03-vsp01.home.lab -vsru admin -vsrp ******** \
    -varf vcf-m03-auto-platform.home.lab -varu admin -varp ******** \
    -fd

      
Authenticating with VSP platform (vcf-m03-vsp01.home.lab)...
Authentication successful.
Authenticating with VSP platform (vcf-m03-auto-platform.home.lab)...
Authentication successful.
Deleting component '12f80ada-c354-4b4d-9a19-6318ce58fe49' from Fleet...
Delete task initiated (ID: 01a1146d-a9ae-75ae-ad6d-c4bcf0904371)
Waiting for task 01a1146d-a9ae-75ae-ad6d-c4bcf0904371 to complete (timeout: 7200 seconds)...
Task status: RUNNING | Elapsed time: 0s
Task status: RUNNING | Elapsed time: 30s
Task status: SUCCEEDED | Elapsed time: 60s
Successfully deleted component '12f80ada-c354-4b4d-9a19-6318ce58fe49' from Fleet.
Waiting 120 seconds before checking if component 12f80ada-c354-4b4d-9a19-6318ce58fe49 has been successfully deleted from Fleet...
Component '12f80ada-c354-4b4d-9a19-6318ce58fe49' does not exist in Fleet (HTTP 404).
Component '12f80ada-c354-4b4d-9a19-6318ce58fe49' does not exist in VSP (HTTP 404).
Checking Consumption VSP (vcf-m03-auto-platform.home.lab) for component...
Authenticating with VSP platform (vcf-m03-auto-platform.home.lab)...
Authentication successful.
Deleting component '12f80ada-c354-4b4d-9a19-6318ce58fe49' from VSP...
Delete task initiated (ID: ybtawneti5c5zecrovul22l42u)
Waiting for task ybtawneti5c5zecrovul22l42u to complete (timeout: 7200 seconds)...
Task status: PENDING | Elapsed time: 0s
Task status: RUNNING | Elapsed time: 30s
    …(中間 37 次 RUNNING 省略)…
Task status: RUNNING | Elapsed time: 1170s
Task status: SUCCEEDED | Elapsed time: 1200s
Successfully deleted component '12f80ada-c354-4b4d-9a19-6318ce58fe49' from VSP.

      
real	23m3.064s
user	0m0.381s
sys	0m0.159s

流程:先刪 Fleet(60 秒)→ 等 120 秒回頭確認 404 → 查管理 VSP(不在,跳過)→ 到 VCFA runtime 刪除(1200 秒)。

## VCFA runtime 上的刪除任務

從 VCFA runtime 的 /api/v1/tasks 查到的任務,三個 stage:

{
    "cancellable": false,
    "createTime": "2026-10-07T03:37:54Z",
    "createdBy": "admin@vsp.local",
    "endTime": null,
    "id": "ybtawneti5c5zecrovul22l42u",
    "messages": [],
    "output": null,
    "phase": "Running",
    "precheckGroups": [],
    "resource": {
        "componentId": "12f80ada-c354-4b4d-9a19-6318ce58fe49"
    },
    "result": null,
    "retryable": true,
    "stages": [
        {
            "description": "Running Delete Component Prechecks",
            "name": "delete-component",
            "stageType": "Precheck",
            "status": "Succeeded"
        },
        {
            "description": "VCF Component Delete Initialization",
            "name": "delete-component-initialization",
            "stageType": "Workflow",
            "status": "Succeeded"
        },
        {
            "description": "Delete VCF Component",
            "name": "delete-component",
            "stageType": "Workflow",
            "status": "Running"
        }
    ],
    "startTime": "2026-10-07T03:37:55Z",
    "status": "Running",
    "type": "com.vmware.vcfms.task.DeleteComponent"
}

## 卸載過程

每分鐘記一次 VCFA runtime 上 prelude(VCFA 本體)namespace 的 pod 狀態,可以看到 pod 逐步進入 CreateContainerConfigError / ContainerCreating 後被清空:

# VCFA 卸載時間軸(每 60 秒)
# 時間     任務狀態    stage3      prelude(R/C/其他)  vcd-migrator  cleanup 程序
11:49:09  Running Running        8 Completed       2 CreateContainerConfigError      60 Running  | 2  running
11:50:18  Running Running        8 Completed       1 CreateContainerConfigError      46 Running      15 Terminating  | 2  running
11:51:25  Running Running        8 Completed       2 Running       3 Terminating  | 2  running
11:52:30  Running Running        2 ContainerCreating  | 2  running
11:53:35  Running Running   | 2  running
11:54:40  Running Running   | 2  running
11:55:46  Running Running   | 2  running
11:56:50  Running Running   | 2  running
11:57:55  Running Running   | 2  running
11:59:00  Running Running   | 2  running
12:00:06  ? ?   | 2  running
12:01:11  ? ?   | 2  running
12:02:16  ? ?   | 2  running
12:03:21  ? ?   | 2  running
12:04:27  ? ?   | 2  running
12:05:33  ? ?   | 2  running
12:06:39  ? ?   | 2  running
12:07:44  ? ?   | 2  running
12:08:49  ? ?   | 1  running
12:09:54  ? ?   | 0  running
12:10:59  ? ?   | 0  running
12:12:04  ? ?   | 0  running
12:13:09  ? ?   | 0  running
12:14:14  ? ?   | 0  running
12:15:18  ? ?   | 0  running
12:16:23  ? ?   | 0  running
12:18:08  ? ?  ?  running
12:19:37  ? ?  ?  running
12:21:06  ? ?  ?  running
12:22:35  ? ?  ?  running
12:24:04  ? ?  ?  running
12:25:33  ? ?  ?  running
12:27:02  ? ?  ?  running
12:28:31  ? ?  ?  running
12:30:00  ? ?  ?  running
12:31:29  ? ?  ?  running

透過 ssh 執行 python 沒有 TTY,stdout 會被緩衝,刪除期間畫面完全沒輸出。建議改用 python3 -u。

# 九、Step 5 — ② 刪除 Migration service engine 元件

① 進行中如果跑 list vsp-component,會暫時看不到 VCD_MIGRATOR —— 但它並沒有被刪掉。下面的輸出顯示 ② 仍在 Fleet 中找到它並執行刪除(Deleting component ... from Fleet → SUCCEEDED)。不要因為 list 看不到就跳過這一步。

# Step ② — 刪除 VCD_MIGRATOR(Migration service engine)元件  2026-10-07 12:01:09
$ python3 -u cleanup_component.py delete vsp-component \
    -c 9c064f80-f259-4c50-98a8-97fbccf26cdc \
    -ff vcf-m03-fleet01.home.lab \
    -vsrf vcf-m03-vsp01.home.lab -vsru admin -vsrp ******** \
    -varf vcf-m03-auto-platform.home.lab -varu admin -varp ******** \
    -fd

      
Authenticating with VSP platform (vcf-m03-vsp01.home.lab)...
Authentication successful.
Authenticating with VSP platform (vcf-m03-auto-platform.home.lab)...
Authentication successful.
Deleting component '9c064f80-f259-4c50-98a8-97fbccf26cdc' from Fleet...
Delete task initiated (ID: 01a11483-5498-7572-9d10-21a29d8cf4c5)
Waiting for task 01a11483-5498-7572-9d10-21a29d8cf4c5 to complete (timeout: 7200 seconds)...
Task status: RUNNING | Elapsed time: 0s
Task status: RUNNING | Elapsed time: 30s
Task status: SUCCEEDED | Elapsed time: 60s
Successfully deleted component '9c064f80-f259-4c50-98a8-97fbccf26cdc' from Fleet.
Waiting 120 seconds before checking if component 9c064f80-f259-4c50-98a8-97fbccf26cdc has been successfully deleted from Fleet...
Component '9c064f80-f259-4c50-98a8-97fbccf26cdc' does not exist in Fleet (HTTP 404).
Component '9c064f80-f259-4c50-98a8-97fbccf26cdc' does not exist in VSP (HTTP 404).
Checking Consumption VSP (vcf-m03-auto-platform.home.lab) for component...
Authenticating with VSP platform (vcf-m03-auto-platform.home.lab)...
Authentication successful.
Deleting component '9c064f80-f259-4c50-98a8-97fbccf26cdc' from VSP...
Delete task initiated (ID: mndfw6dezbf5lcbnrfcouytlgi)
Waiting for task mndfw6dezbf5lcbnrfcouytlgi to complete (timeout: 7200 seconds)...
Task status: PENDING | Elapsed time: 0s
Task status: RUNNING | Elapsed time: 30s
    …(中間 15 次 RUNNING 省略)…
Task status: RUNNING | Elapsed time: 510s
Task status: SUCCEEDED | Elapsed time: 540s
Successfully deleted component '9c064f80-f259-4c50-98a8-97fbccf26cdc' from VSP.

      
real	12m1.873s
user	0m0.258s
sys	0m0.135s

刪除後再 list 一次,VCFA runtime 上已經沒有任何元件,只剩管理 VSP 上的 Identity broker:

# Step ③ 之前 — 確認 VCFA runtime 上已經沒有元件  12:13:29
$ python3 -u cleanup_component.py list vsp-component ...
Authenticating with VSP platform (vcf-m03-vsp01.home.lab)...
Authentication successful.
Fetching VSP components from Fleet...

      
Available VSP Components:
------------------------------------------------------------------------------------------------
ID | Type | Deployment | Version | FQDN | VCF Services Runtime FQDN
------------------------------------------------------------------------------------------------
fe04c14c-4c68-4db4-b91b-05006adf5139 | Identity broker | VSP | 9.1.1.0.25679886 | vcf-m03-vidb.home.lab | vcf-m03-vsp01.home.lab
------------------------------------------------------------------------------------------------

      
$ python3 -u cleanup_component.py list vsp-cluster ...
Authenticating with VSP platform (vcf-m03-vsp01.home.lab)...
Authentication successful.
Fetching VSP components from Fleet...

      
Available VSP Components:
------------------------------------------------------------------------------------------------
ID | Type | Deployment | Version | FQDN | VCF Services Runtime FQDN
------------------------------------------------------------------------------------------------
4fde9186-f3d4-48d6-ab64-a06664dad047 | VCF services runtime | VSP | 9.1.1.0.25714471 | vcf-m03-auto-platform.home.lab | vcf-m03-auto-platform.home.lab
------------------------------------------------------------------------------------------------

# 十、Step 6 — 在 SDDC Manager 取得 root

③ 會直接改 SDDC Manager 的 platform 資料庫,腳本會檢查兩件事:os.geteuid() == 0,以及 http://localhost/commonsvcs/about 回傳 name=COMMON_SERVICES。

vcf 帳號不在 sudoers,su 又必須在終端機裡執行。用 python 的 pty 模組開虛擬終端、看到密碼提示就送出密碼:

# Step ③-準備 — 在 SDDC Manager 取得 root
$ sudo -n id                 # vcf 不在 sudoers
sudo: a password is required

      
$ su - root -c id           # 沒有 TTY 會失敗
su: must be run from a terminal

      
$ ROOTPW=******** python3 asroot.py "id; curl -s http://localhost/commonsvcs/about"

      
uid=0(root) gid=0(root) groups=0(root)
{"id":"59279484-a78b-4269-8f84-3b4919d5d533","name":"COMMON_SERVICES","version":"9.1.1.0.25713929","status":"ACTIVE","serviceUrl":"http://localhost/commonsvcs","description":"Platform services"}

# asroot.py(核心邏輯)
pid, fd = pty.fork()
if pid == 0:
    os.execvp("su", ["su", "-", "root", "-c", cmd])
# 父程序:讀到 "assword" 就寫入 root 密碼 + 換行,其餘輸出照印

# 十一、Step 7 — ③ 刪除 VCFA services runtime

這一步要給 vCenter 帳密(-vu / -vp),腳本會從 Fleet 取得 runtime 的 VM 清單與所在 vCenter,然後關機刪除。

# Step ③ — 刪除 VCFA VCF services runtime(必須 root)  2026-10-07 12:13:50
# python3 -u cleanup_component.py delete vsp-cluster \
    -c 4fde9186-f3d4-48d6-ab64-a06664dad047 \
    -ff vcf-m03-fleet01.home.lab \
    -vsrf vcf-m03-vsp01.home.lab -vsru admin -vsrp ******** \
    -vu administrator@vsphere.local -vp ******** \
    -fd

      

      
whoami: root   time: 2026-10-07 04:11:13
Verifying this script is running as root on the SDDC Manager VM...
Verified: running as root on the SDDC Manager VM.

      
Authenticating with VSP platform (vcf-m03-vsp01.home.lab)...
Authentication successful.
Fetching component '4fde9186-f3d4-48d6-ab64-a06664dad047' details from Fleet...
Component has 1 VM node(s): vcf-m03-auto-platform-lmh4w
Hosting vCenter: vcf-m03-vc01.home.lab
Authenticating with vCenter (vcf-m03-vc01.home.lab)...
vCenter authentication successful.
Deleting component '4fde9186-f3d4-48d6-ab64-a06664dad047' from Fleet...
Delete task initiated (ID: 01a1148e-eff4-7648-84ba-92d0faa51ee9)
Waiting for task 01a1148e-eff4-7648-84ba-92d0faa51ee9 to complete (timeout: 7200 seconds)...
Task status: RUNNING | Elapsed time: 0s
Task status: RUNNING | Elapsed time: 30s
Task status: SUCCEEDED | Elapsed time: 60s
Successfully deleted component '4fde9186-f3d4-48d6-ab64-a06664dad047' from Fleet.
Waiting 120 seconds before checking if component 4fde9186-f3d4-48d6-ab64-a06664dad047 has been successfully deleted from Fleet...
Component '4fde9186-f3d4-48d6-ab64-a06664dad047' does not exist in Fleet (HTTP 404).
Authenticating with vCenter (vcf-m03-vc01.home.lab)...
vCenter authentication successful.
Looking up VM 'vcf-m03-auto-platform-lmh4w' in vCenter...
Found VM 'vcf-m03-auto-platform-lmh4w' with identifier 'vm-43'.
Checking power state for VM 'vcf-m03-auto-platform-lmh4w' (vm-43)...
Powering off VM 'vcf-m03-auto-platform-lmh4w'...
Successfully powered off VM 'vcf-m03-auto-platform-lmh4w'.
Deleting VM 'vcf-m03-auto-platform-lmh4w' (vm-43) from vCenter...
Successfully deleted VM 'vcf-m03-auto-platform-lmh4w' (vm-43) from vCenter.
Deleting VSP cluster '4fde9186-f3d4-48d6-ab64-a06664dad047' from the platform database...
Successfully deleted VSP cluster '4fde9186-f3d4-48d6-ab64-a06664dad047' from the platform database.
psql output: DO

      
real	3m10.212s
user	0m0.265s
sys	0m0.211s

| 動作 | 結果 |
| --- | --- |
| 驗證 root | Verified: running as root on the SDDC Manager VM |
| Fleet 刪除 | task 01a1148e… SUCCEEDED(60 秒),120 秒後確認 404 |
| vCenter | vm-43 vcf-m03-auto-platform-lmh4w 關機 → 刪除 |
| SDDC Manager platform DB | vsp_cluster 紀錄刪除(psql: DO) |

# 十二、Step 8 — 移除後驗證

圖 4　移除後:Fleet 元件剩 9 個,VCF Automation 已不在清單

圖 5　移除後:Fleet → Tasks 留有三筆刪除紀錄,狀態皆為 Completed

Delete VCF Automation component           12f80ada…  Completed
Delete Migration service engine component 9c064f80…  Completed
Delete VCF services runtime component     4fde9186…  Completed

圖 6　移除後:VCFA console 連線逾時(net::ERR_CONNECTION_TIMED_OUT)

# 移除後狀態 — 2026-10-07 12:17:18

      
## vCenter(vcf-m03-vc01)上的 VM
  vcf-m03-lic01                                     2 vCPU    4 GB  10.0.1.62    poweredOn
  vcf-m03-nsx01a                                    6 vCPU   24 GB  10.0.1.64    poweredOn
  vcf-m03-ops01                                     4 vCPU   16 GB  10.0.1.61    poweredOn
  vcf-m03-opsc01                                    4 vCPU   16 GB  10.0.1.66    poweredOn
  vcf-m03-sddcm01                                   4 vCPU   16 GB  10.0.1.65    poweredOn
  vcf-m03-vc01                                      4 vCPU   21 GB  10.0.1.60    poweredOn
  vcf-m03-vsp01-fz8hf                               4 vCPU   10 GB  10.0.1.82    poweredOn
  vcf-m03-vsp01-g7m86                              12 vCPU   24 GB  10.0.1.83    poweredOn
  vcf-m03-vsp01-kdwcz                              12 vCPU   24 GB  10.0.1.84    poweredOn
  vcf-services-runtime-template-9.1.1.0.25714471    6 vCPU   24 GB  -            poweredOff

      
## 端點
  https://vcf-m03-auto-vip.home.lab/automation/      000
  https://vcf-m03-auto-vip.home.lab/provider         000
  https://vcf-m03-vidb.home.lab/                     404
  https://vcf-m03-ops01.home.lab/                    200
  (000 = 連不上)

      
## DNS(腳本不會動 DNS)
  vcf-m03-auto-vip           10.0.1.72
  vcf-m03-auto-platform      10.0.1.73

      
## SDDC Manager platform DB:vsp_cluster

      
1
16fbc3aa-0413-4df2-adc0-a8534bbe01f4
  ↑ 第一行=筆數,之後=剩下的 vsp_cluster_id(4fde9186… 應該不在)

      
## Identity broker 是否受影響(管理 VSP 上的 vidb pod)
NAMESPACE         NAME                 AGE   PHASE        STATUS
salt-raas         salt-raas            15h   Successful   successful package deployment
salt              salt                 15h   Successful   successful package deployment
telemetry         telemetry-acceptor   16h   Successful   successful package deployment
vcf-fleet-depot   vcf-fleet-depot      18h   Successful   successful package deployment
vcf-fleet-lcm     vcf-fleet-lcm        18h   Successful   successful package deployment
vcf-sddc-lcm      vcf-sddc-lcm         18h   Successful   successful package deployment
vidb-external     vidb                 16h   Successful   successful package deployment
vmsp-platform     vmsp-platform        19h   Successful   successful package deployment

      
      2 Running

      
## ping 10.0.1.98 判讀
  Reply from 10.0.0.200: Destination host unreachable.   ← 本機自己回的,不是目標
  Test-Connection 3 次全 TimedOut                          ← 10.0.1.98 確實已不存在
  ⚠ Windows ping 會把「Destination host unreachable」算進 Received,不能只看收到幾個

| 檢查項目 | 移除前 | 移除後 |
| --- | --- | --- |
| Fleet 元件數 | 10 | 9(VCF Automation 消失) |
| vCenter VM | 11(含 auto-platform-lmh4w) | 10(auto-platform-lmh4w 已刪) |
| VCFA /automation/ | 200 | 000(連不上) |
| 10.0.1.98(runtime 節點) | 有回應 | Test-Connection 全 TimedOut |
| platform DB vsp_cluster | 含 4fde9186…(移除前未直接查;③ 刪除時腳本確認它存在並刪除) | 1 筆,只剩管理 VSP 的 16fbc3aa… |
| Identity broker(管理 VSP) | pd Successful | pd Successful、2 pod Running(未受影響) |
| DNS auto-vip / auto-platform | 存在 | 仍存在(腳本不處理 DNS) |

# 十三、注意事項與坑

| 症狀 / 情境 | 原因 | 做法 |
| --- | --- | --- |
| Failed to obtain VSP token (HTTP 401) | 用了 administrator@vsphere.local | VSP 帳號一律用 admin |
| ① 進行中 list 看不到 VCD_MIGRATOR | Fleet 清單暫時性的狀態 | 不要跳過 ②;③ 之前再 list 確認 |
| 直接跑 ③ 會留下孤兒 | 腳本不檢查 runtime 上是否還有元件 | 嚴守 ① → ② → ③ 順序 |
| ③ 報 must be root | vsp-cluster 刪除要改本機 DB | 用 pty 執行 su(見 Step 6) |
| 刪除期間畫面沒輸出 | ssh 無 TTY,python stdout 緩衝 | python3 -u |
| 元件版本 < 9.1 | should_skip_infra_deletion 會跳過實體刪除 | 只會清 Fleet 帳面,VM 要另外處理 |
| ping 看起來有回應 | Windows 把 Destination host unreachable 算成 Received | 看回應來源,或用 Test-Connection |
| DNS 記錄還在 | 腳本不處理 DNS | 自行刪除 auto-vip / auto-platform 的 A 與 PTR |

已知後續問題(KB 441127):VCFA 元件移除後若要再加回,可能因 infrastructure properties 為 NULL 而無法 import。移除前請確認之後不會需要原地加回同一套。

# 附錄 A:可直接複製的完整指令

以下在 SDDC Manager 上執行;密碼以變數代替。

# 0) 準備
cd /home/vcf
export VSPPW='<VSP admin 密碼>'  VCPW='<vCenter administrator@vsphere.local 密碼>'
FLEET=vcf-m03-fleet01.home.lab
MGMT_VSP=vcf-m03-vsp01.home.lab
VCFA_VSP=vcf-m03-auto-platform.home.lab

      
# 1) 列出 ID(唯讀)
python3 -u cleanup_component.py list vsp-component -ff $FLEET -vsrf $MGMT_VSP -vsru admin -vsrp "$VSPPW"
python3 -u cleanup_component.py list vsp-cluster   -ff $FLEET -vsrf $MGMT_VSP -vsru admin -vsrp "$VSPPW"

      
# 2) ① 刪 VCF Automation(vcf 帳號即可)
python3 -u cleanup_component.py delete vsp-component -c <VCFA_ID> \
  -ff $FLEET -vsrf $MGMT_VSP -vsru admin -vsrp "$VSPPW" \
  -varf $VCFA_VSP -varu admin -varp "$VSPPW" -fd

      
# 3) ② 刪 Migration service engine(VCD_MIGRATOR)
python3 -u cleanup_component.py delete vsp-component -c <VCD_MIGRATOR_ID> \
  -ff $FLEET -vsrf $MGMT_VSP -vsru admin -vsrp "$VSPPW" \
  -varf $VCFA_VSP -varu admin -varp "$VSPPW" -fd

      
# 4) 再 list 一次,確認 VCFA runtime 上已經沒有元件
python3 -u cleanup_component.py list vsp-component -ff $FLEET -vsrf $MGMT_VSP -vsru admin -vsrp "$VSPPW"

      
# 5) ③ 刪 VCFA services runtime(必須 root)
python3 -u cleanup_component.py delete vsp-cluster -c <VSP_CLUSTER_ID> \
  -ff $FLEET -vsrf $MGMT_VSP -vsru admin -vsrp "$VSPPW" \
  -vu administrator@vsphere.local -vp "$VCPW" -fd

# 附錄 B:腳本參數一覽

| 參數 | 長名稱 | 用在 | 說明 |
| --- | --- | --- | --- |
| -ff | --fleet-fqdn | 全部 | Fleet lifecycle 的 FQDN |
| -vsrf | --vcf-services-runtime-fqdn | 全部 | 管理 VSP 的 FQDN |
| -vsru / -vsrp | --vcf-services-runtime-username/password | 全部 | 管理 VSP 帳密(帳號用 admin) |
| -c | --component-id | delete | 要刪的元件或 runtime ID |
| -fd | --force-delete | delete | 不詢問直接刪 |
| -varf | --vcfa-vcf-services-runtime-fqdn | delete vsp-component | VCFA runtime FQDN(刪 VCFA / Migration service engine 必填) |
| -varu / -varp | --vcfa-vcf-services-runtime-username/password | delete vsp-component | VCFA runtime 帳密(帳號用 admin) |
| -vu / -vp | --vcenter-username/password | delete vsp-cluster | vCenter 帳密,用來關機刪 VM |

| 常數 | 值 | 意義 |
| --- | --- | --- |
| POLL_INTERVAL_IN_SECONDS | 30 | 每 30 秒查一次任務狀態 |
| MAX_POLL_TIME_IN_SECONDS | 7200 | 單一任務最多等 2 小時 |
| FLEET_DELETE_RETRY_DELAY_IN_SECONDS | 120 | Fleet 刪完後等 2 分鐘回頭確認 |